1. Two finger grippers are good for ________________. a.; side-to-side centering. b.; centering a part in two directions at once. c.; odd-shaped parts. d.; long parts. ANSWER:; a. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

2. Five finger grippers are recommended for ________________. a.; side-to-side centering. b.; centering a part in two directions at once. c.; odd-shaped parts. d.; simple round parts. ANSWER:; c. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

3. ________ is the force that an object pushes back with when acted on by a force. a.; Friction. b.; Magnetic force. c.; Normal force. d.; Vacuum force. ANSWER:; c. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

4. _____ grippers have fingers that hinge or pivot on a point to move the tips of the fingers. a.; Angular. b.; Parallel. c.; Magnetic. d.; RCC. ANSWER:; a. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

5. _____ grippers have fingers that move in straight lines toward the center or outside of the part. a.; Angular. b.; Parallel. c.; Magnetic. d.; RCC. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

6. Which of these is where we consider the mass to be centered and all forces in equilibrium? a.; Normal force. b.; Safety factor. c.; Center of gravity. d.; RCC. ANSWER:; c. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

7. MIG welding systems use ___________ to fuse parts. a.; intense beams of light. b.; electricity and feed wire. c.; high-frequency vibration. d.; current passing from one tip to the other. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

8. Ultrasonic welding systems use ___________ to fuse parts. a.; intense beams of light. b.; electricity and feed wire. c.; high-frequency vibration. d.; current passing from one tip to the other. ANSWER:; c. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

9. Laser welding systems use ___________ to fuse parts. a.; intense beams of light. b.; electricity and feed wire. c.; high-frequency vibration. d.; current passing from one tip to the other. ANSWER:; a. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

10. Active remote center compliance systems ____________. a.; incorporate sensors into the system for feedback. b.; use shear plates and springs to allow for part misalignment. c.; are part of an open-loop control system. d.; All of these are correct. ANSWER:; a. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

11. A(n) ___________ for spot welding consists of a fixed tip and a movable welding tip. a.; X-type gun. b.; C-type gun. c.; RCC- type gun. d.; welding gun. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

12. A(n) ___________ is a tube-like tool that people or robots use to direct the electrically charged wire used during welding operations. a.; X-type gun. b.; C-type gun. c.; RCC- type gun. d.; welding gun. ANSWER:; d. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

13. ____________ grippers work by creating a pressure that is less than the atmospheric pressure around the system. a.; Magnetic. b.; Balloon. c.; Vacuum. d.; Pin. ANSWER:; c. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

14. ____________ grippers work by using inflation or deflation to grip the part. a.; Magnetic. b.; Balloon. c.; Vacuum. d.; RCC. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

15. What is the difference between angular grippers and parallel grippers? ANSWER:; Parallel grippers have fingers that move in straight lines toward the center or outside of the part to close and grip or open and release the part, respectively. Angular grippers have fingers that hinge or pivot on a point to move the tips outward to release parts or inward to grip parts.. POINTS:; 2. QUESTION TYPE:; Subjective Short Answer. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

16. Describe the process of spot welding. ANSWER:; Spot welding uses a device called a gun, which resembles a large copper clamp, to fuse metal together by passing current from one tip to the other, through the material fused.. POINTS:; 2. QUESTION TYPE:; Subjective Short Answer. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

17. What is the difference between active and passive RCC units? ANSWER:; Active RCCs have sensors inside the unit to detect how much side shift is occurring, whereas passive RCCs use springs and shear plates.. POINTS:; 2. QUESTION TYPE:; Subjective Short Answer. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

18. How do vision systems help with part misalignment or variance in parts? ANSWER:; With vision systems, the robot uses a camera to take a picture of the part and then compares this picture to sorting criteria predefined by the programmer. Once the system has filtered the visual information, it runs a subroutine that allows it to offset the tooling positioning in relation to the new part position.. POINTS:; 2. QUESTION TYPE:; Subjective Short Answer. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

19. What are the four rules for selecting a gripper and how are they commonly used by industry in the gripper selection process? Make sure to fully support your answer. ANSWER:; 1 The tooling must be capable of holding, centering, or manipulating the range of parts the robot will work with. 2 There needs to be some way of sensing when a gripper has closed on the part or if it is empty. 3 The weight of the gripper should be kept as light as possible as this weight is deducted from the robots total payload. 4 Proper safety features built into the gripper for the environment it will work in and the parts it will work with.. POINTS:; 3. QUESTION TYPE:; Essay. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.

20. How does a Venturi valve work? ANSWER:; For these systems, we pass compressed air through a conical Venturi orifice, as the air passes from the restriction to the larger area, the pressure falls and the velocity increases. Because of the large difference between the conical restriction and the line after it, an intense reaction is created that sucks the stationary air out of the area of the vacuum cup or wherever the air opening is attached into the main line after the Venturi valve.. POINTS:; 3. QUESTION TYPE:; Essay. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:46 AM. DATE MODIFIED:; 6/11/2018 9:46 AM.