1. Micro limit switches commonly have an amperage range of ___________. a.; 1 to 3 amps. b.; 1 to 7 amps. c.; 1 to 10 amps. d.; 1 to 20 amps. ANSWER:; d. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

2. _______ are made up of a solid piece of material that manipulates the flow of electrons without any moving parts. a.; Limit switches. b.; Solid-state devices. c.; Optical encoders. d.; Target flowmeters. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

3. You are setting up a system to detect if fluid is inside a carton so the robot can pick the empty cartons off the line. Which of the following would work for this task? a.; Inductive proximity switch. b.; Capacitive proximity switch. c.; Photoelectric proximity switch. d.; Hall effect sensors. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

4. _________________ are devices activated by contact with an object that changes the state of its contacts when the object exerts a certain amount of force. a.; Limit switches. b.; Inductive proximity switches. c.; Photoelectric proximity switches. d.; Hall effect sensors. ANSWER:; a. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

5. _________________use an oscillating magnetic field to detect ferrous metal items. a.; Limit switches. b.; Inductive proximity switches. c.; Photoelectric proximity switches. d.; Hall effect sensors. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

6. _____________ photoelectric sensors separate the transmitter and receiver into different units, placed opposite each other. a.; Through-beam. b.; Retroreflective. c.; Diffuse. d.; Light-level detection. ANSWER:; a. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

7. _____________ photoelectric sensors place the transmitter and receiver in one assembly and often use something like a bicycle reflector to return the light. a.; Through-beam. b.; Retroreflective. c.; Diffuse. d.; Light-level detection. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

8. When using the amperage draw of the motors to monitor for impact, which of the following statements is true? a.; These systems are complex and require high-level math.. b.; Noise in the system may cause a false shut down.. c.; Excessive payload weight may cause the system to shut down.. d.; All of these are correct.. ANSWER:; d. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

9. __________ are based on the principle of a linear increase in resistance that occurs when a metal is exposed to heat. a.; Thermocouples. b.; RTDs. c.; Thermistors. d.; IR temperature sensors. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

10. __________ are temperature sensors that work off the principle of a small DC millivoltage generated when the junction of two dissimilar metals are heated. a.; Thermocouples. b.; RTDs. c.; Thermistors. d.; IR temperature sensors. ANSWER:; a. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

11. __________ place a disk or similar shape in the fluid and use the force of the fluid flow to deflect the disk and generate a signal. a.; Turbine flowmeters. b.; Target flowmeters. c.; Sail flowmeters. d.; Magnetic flowmeters. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

12. __________ use a propeller-type assembly placed inside the fluid to generate a signal via a magnetic pickup sensor positioned nearby, but outside of the fluid. a.; Turbine flowmeters. b.; Target flowmeters. c.; Sail flowmeters. d.; Magnetic flowmeters. ANSWER:; a. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

13. _____________ use a thin membrane, often made of metal in a capsule arrangement, to move the indicator assembly. a.; Bourdon tube gauges. b.; Bellows gauges. c.; Diaphragm gauges. d.; Pressure relief valves. ANSWER:; c. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

14. _____________ use a thin-walled, slightly elliptical, cross-sectioned tube bent in a C shape, which is tied directly to the system, to read pressure. a.; Bourdon tube gauges. b.; Bellows gauges. c.; Diaphragm gauges. d.; Pressure relief valves. ANSWER:; a. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

15. ___________ are similar to photo eyes except that they emit and receive sound instead of light. a.; Sourcing signals. b.; Sinking signals. c.; Optical isolation. d.; Ultrasonic sensors. ANSWER:; d. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

16. ___________ provide(s) the positive connection or DC voltage to the input module. a.; Sourcing signals. b.; Sinking signals. c.; Optical isolation. d.; Ultrasonic sensors. ANSWER:; a. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

17. What is the difference between a sourcing and a sinking signal? ANSWER:; A sourcing signal input provides the positive connection or DC voltage (usually 24 V) to the input module. A sinking signal input provides the negative connection or ground to the input module.. POINTS:; 2. QUESTION TYPE:; Subjective Short Answer. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

18. How does an absolute encoder work? ANSWER:; Absolute optical encoders add enough emitters and receivers, usually four or more in total, to give each position of the encoder its own unique binary address.. POINTS:; 2. QUESTION TYPE:; Subjective Short Answer. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

19. How do thermocouples work? ANSWER:; Thermocouples are temperature sensors that work off the principle of a small DC millivoltage generated when the junction of two dissimilar metals is heated.. POINTS:; 2. QUESTION TYPE:; Subjective Short Answer. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

20. What is the benefit of disengaging the axes as opposed to the E-stop method of stopping the robot when impact is detected? ANSWER:; It reduces the force the robot has to stop, the energy of the impact, and the stress on the robot’s internal systems.. POINTS:; 2. QUESTION TYPE:; Subjective Short Answer. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

21. How do we commonly monitor impact today? ANSWER:; Today we commonly either monitor the amperage the motor uses or insert sensing devices designed to detect impacts. The common sensing devices are strain gauges and the like at crucial points, such as the joints of the robot, to detect sudden changes in the forces of motion so the robot can respond accordingly.. POINTS:; 2. QUESTION TYPE:; Subjective Short Answer. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.

22. What are the six main criteria for selecting a sensor? ANSWER:; • What are you measuring • Operating environment • Failsafe requirements • Accuracy • Type of output • Cost versus performance. POINTS:; 3. QUESTION TYPE:; Essay. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 9:57 AM. DATE MODIFIED:; 6/11/2018 9:57 AM.