1. Networks are used to _________________. a.; transmit data. b.; allow for facility remote maintenance. c.; share resources. d.; All of these are correct. ANSWER:; d. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

2. Which topology has all the nodes connected in series to form a continuous loop? a.; Ring. b.; Mesh. c.; Bus. d.; Star. ANSWER:; a. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

3. Which topology has one main cable, called a trunk cable, with terminating resistors at each end? a.; Star. b.; Bus. c.; Mesh. d.; Tree. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

4. Which topology has multiple connections where each node is connected to all or several nodes? a.; Mesh. b.; Ring. c.; Star. d.; Bus. ANSWER:; a. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

5. Which topology uses a central connection which has connections leading to each separate device on the network? a.; Tree. b.; Star. c.; Bus. d.; Mesh. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

6. We refer to induced voltage from magnetic fields or radio waves as ___________. a.; half duplex. b.; simplex. c.; shielding. d.; noise. ANSWER:; d. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

7. ___________ is a generic term for a number of industrial network protocols designed to provide real-time control of devices. a.; Ethernet/IP. b.; Fieldbus. c.; DeviceNet. d.; ControlNet. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

8. ___________ is a deterministic protocol where we can set the minimum rate of data transfer within given parameters. a.; Fieldbus. b.; ControlNet. c.; DeviceNet. d.; Ethernet/IP. ANSWER:; b. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

9. ___________ uses the multi-drop type of network with multiple devices, with their own GSD file, on a single cable. a.; ControlNet. b.; Ethernet/IP. c.; Profibus. d.; DeviceNet. ANSWER:; c. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

10. Which of the following is not a part of the Profibus protocol family? a.; Profibus Decentralized Periphery. b.; Profibus PA. c.; Profibus FMS. d.; None of these are correct. ANSWER:; d. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

11. Which of the following was designed specifically for the process industry? a.; Profibus FMS. b.; Profibus DP. c.; Profibus PA. d.; ProfiNet. ANSWER:; c. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

12. Which of the following is used for communicating at the cell level with devices such as PLCs and industrial-grade PCs? a.; Profibus FMS. b.; Profibus DP. c.; Profibus PA. d.; ProfiNet. ANSWER:; a. POINTS:; 1. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

13. How does producer/consumer communication work? ANSWER:; Producer/consumer communication method is where a device “produces”’ or sends information with specific identifiers attached and other devices “consume” the data that match the identifiers they are looking for. This method can allow multiple devices to consume a single produced transmission.. POINTS:; 1. QUESTION TYPE:; Subjective Short Answer. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

14. How does Peer-to-Peer communication work? ANSWER:; Peer-to-Peer (P2P) communication is where each node can talk directly to one another without having to go through a third node, such as the main controller, and act as sender or receiver of data.. POINTS:; 1. QUESTION TYPE:; Subjective Short Answer. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

15. How does polling data transfer work? ANSWER:; Polling controls communication via the master or requesting unit sending a signal asking slave or sending unit if it has and data to send. If the answer is yes, the data are transmitted from the sending/slave unit. If the answer is no, the master/requesting unit moves on to the next node.. POINTS:; 2. QUESTION TYPE:; Subjective Short Answer. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

16. How does master/slave communication work? ANSWER:; In master/slave communication, the controller, the master, asks for data from nodes, the slaves, and they only respond when directed. This method ensures clean communication and allows for precise timing of messages.. POINTS:; 2. QUESTION TYPE:; Subjective Short Answer. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 10:04 AM.

17. What are the five main things to keep in mind when working to integrate systems? ANSWER:; · The process involved · the equipment involved · The protocol(s) used · The data tracked · The work environment. POINTS:; 3. QUESTION TYPE:; Essay. HAS VARIABLES:; False. DATE CREATED:; 6/11/2018 10:04 AM. DATE MODIFIED:; 6/11/2018 12:40 PM.